# The Fort – Drone Data Collection Program

### The following is information related to this project.

I commissioned Bush Drone Data, LLC (Austin and Dakota Bush) to perform a drone scan of the property at  896 Rock Spring Rd, Owens Cross Roads AL, 35763 (34.570904°, -86.475557°). Austin and Dakota executed this commission on the morning of December 9, 2025. Following is a description of the products I received for this service, all are geo-rectified and we will be working with each a lot:

- Aerial Images: A folder containing a set of 824 24 bit RGB images, size 5280 x 3956, each with the GPS location encoded in their header

- Digital Terrain Models: A folder containing the following:

  - RockSpringRd_DTM_1ft_elev.tif: 1 foot resolution l Terrain Model (DTM) of the property

  - RockSpringRd_DTM_1inch_elev.tif: 1 inch resolution DTM of the property

  - RockSpringRd_DTM_1inch_elev.tif.aux.xml Unknown, see uploaded file of the same name

  - RockSpringRd_DTM_6inch_elev.tif: 6 inch resolution l DTM of the property

- Ortho Imagery: A folder containing the following:

  - RockSpringRd_120925_OrthoImage_fullres.prj: Projection file for ortho-rectified imagery (Uploaded)

  - RockSpringRd_120925_OrthoImage_fullres.tfw: Unknown, Uploaded

  - RockSpringRd_120925_OrthoImage_fullres.tif: Full resolution (2cm I believe) ortho-rectified imagery of the property

  - RockSpringRd_120925_OrthoImage_fullres.tif.aux.xml: Unknown, Uploaded

- Point Cloud Data: A folder containing the following:

  - RockSpringRd_PointCloud_fulldensity_120925.las: Lidar point cloud data

- Projection File: A folder containing the following:

  - Nad83Navd88ftus_Alabamaeastftus_projectionfile.prj: Some sort of projection file (Uploaded)

- Notes on Data.txt: Text notes from the service provider (Uploaded)

- Invoice 1471.pdf: Invoice for service (Uploaded)

### What each deliverable is (and what the “unknown” files mean)

#### 1) Aerial Images (824 RGB photos w/ GPS in header)

- These are your original survey photos, with GPS stored in EXIF (“geotagged”).

- Important nuance: the photos’ geotags are WGS84 + ellipsoid, not the same CRS/vertical reference as the derived GIS products.

- Practically: you’ll mostly use these for archival/context, or if you ever want to re-run photogrammetry yourself.

#### 2) Digital Terrain Models (DTMs) — GeoTIFF rasters

- These are “bare earth” elevation grids derived from the LiDAR ground classification.

- You have multiple resolutions (1 ft, 6 in, 1 in) to trade file size/performance vs detail.

- What is *.tif.aux.xml?
It’s a GDAL sidecar metadata file that stores things like raster statistics (min/max/mean/stddev) and sometimes histogram/overviews info. It’s not required, but it’s useful and harmless to keep with the TIFF. For example, your 1-inch DTM aux file contains precomputed statistics (min/max/mean/etc.).

#### 3) Ortho Imagery — GeoTIFF + sidecars

- The orthomosaic was generated from the aerial photos and is georeferenced with ~2.17 cm/pixel resolution.

- What is *.tfw?
A world file: 6 numbers defining the affine transform (pixel size, rotation, and upper-left origin). Some workflows include it even when the GeoTIFF already embeds georeferencing.

- What is *.prj?
A text definition of the coordinate reference system (CRS). In your case, it helps GIS tools lock onto the correct CRS if anything is ambiguous.

- What is *.tif.aux.xml?
Same idea as the DTM: GDAL metadata/statistics per band. Your ortho aux file shows per-band stats for RGB.

#### 4) Point Cloud — .las

- LiDAR point cloud, with an approximate density of 243 points per square US-survey foot.

- Ground classification: points are Ground or Unclassified; the ground class was used to generate the DTMs.

- Caveat (important in rocky/leaf-litter terrain): ground classification is not guaranteed 100% accurate.

#### 5) Projection file (NAD83...Alabamaeast...NAVD88...ftus)

- This is the “truth source” CRS definition for the GIS products. The provider states that all datasets except the aerial-image geotags are referenced to:
NAD83(2011) Alabama East (US survey ft) + NAVD88 (Geoid18) (US survey ft)

- Keep this file with the dataset forever.

#### 6) Notes on Data.txt (provider notes + constraints)

- Key points:

  - Not for legal surveying / boundary surveying use.

  - Sensor/platform: DJI Zenmuse L2 on DJI M350 RTK; RTK via ALDOT CORS; post-processed with PPK + virtual base station.

#### 7) Invoice

- Invoice 1471, dated 12/11/2025, $1,200 total.