# DTI – Rock Spring Rd Property Line Management Process

Here’s a “good enough” field process for using onX on iPhone to place your 90-ft ASoT markers without measuring between trees, while still staying aligned to the ≤100 ft spacing requirement for purple-paint posting on forest land.

## Before you go (one-time setup + day-of checklist):

- Import the markers into onX

- Import PaintMarkers_90ft.kml and put all points in one folder (e.g., ASoT_Paint_90ft).

- Download Offline Maps for the whole boundary corridor

- Do this before you leave cell service. onX explicitly supports offline use, and GPS does not require cell service.

- iPhone location settings (critical):

  - Settings → Privacy & Security → Location Services → ON

  - onX → Location → While Using (or “Always” if you prefer) + Precise Location ON

  - Turn Wi-Fi ON (even if not connected) — it can help location solutions in some conditions.

  - Avoid Low Power Mode while placing points (it can reduce background/location performance).

- Carry stake kit for “no tree within 5 ft”

  - Pre-cut stakes/posts, a hammer, and paint. (Purple can be on trees or posts; spacing/visibility rules still apply.)

### Field process (repeat for every waypoint PNT-###):

- Navigate to the waypoint:

  - Open PNT-### → tap Go to Waypoint (onX shows a straight-line route with distance + direction).

  - Walk toward it normally until you’re within ~30 ft.

- Switch to “tight approach” mode:

  - Tap the home/location button to snap to your blue dot; tap it again to enter Compass Mode (onX calls out the “flashlight” direction indicator).

  - Slow down for the last 30 ft. Keep the phone in your hand, not in a pocket.

- Decide whether the GPS is “good enough”:

  - When you first reach ~10–15 ft, STOP for 20–30 seconds and watch the distance-to-waypoint reading.

  - Use this simple rule:

    - GOOD ENOUGH = the distance reading is stable (not jumping wildly) and you can get it to ≤ 5 ft and keep it there ~10 seconds while standing still.

    - NOT GOOD ENOUGH = while you’re standing still, the distance reading keeps swinging by ~10+ ft, or it won’t settle after ~2 minutes.

- Place the mark:

  - If GOOD ENOUGH:

    - Pick the closest suitable tree within 5 ft (your rule).

    - Paint it (or flag + paint, per your preference).

  - If NOT GOOD ENOUGH (don’t gamble):

    - Place a stake/post at the best-minimum distance point you can achieve:

    - Walk a small 6–10 ft circle around the indicated point and watch the distance value.

    - Stop where it hits its minimum, let it sit ~10 seconds, then set the stake there.

    - This keeps you from “drifting” 15–20 ft off and accidentally breaking the 90+90 spacing buffer.

- Create an audit trail (fast, but valuable):

  - On the waypoint, add:

    - Photo of the painted tree/stake

    - Note, Painted / Stake + date => This makes later maintenance and dispute resolution much easier.

### Why this keeps you compliant (without measuring between marks)

- The Alabama purple paint posting requires marks be readily visible and ≤100 ft apart on forest land (plus size/height rules).

- Your ASoT is 90 ft spacing, so the whole plan only works if each mark stays close to its waypoint (your 5 ft cap).

- The steps above are designed to prevent the two big failure modes: (1) GPS drift under canopy and (2) “tree hunting” far from the waypoint.

If you want, I can also give you a 2-minute “at-home” validation drill (in your yard) to confirm what “GOOD ENOUGH” looks like on your specific iPhone model before you do the whole boundary.

### 2-Minute “Good Enough” GPS Validation (iPhone + onX):

- Prep (15 seconds):

  - Go outside to an open-sky spot (driveway/backyard, away from the house and big trees if possible).

  - Open onX and make sure your blue dot is showing.

- Step 1 — Create a test waypoint (15 seconds):

  - In onX, create a waypoint called TEST-GPS at your current location (drop a pin / add waypoint).

- Step 2 — Stability check (30 seconds):

  - Tap TEST-GPS → Go To.

  - Stand still with the phone held in front of you.

  - Watch the distance-to-waypoint for 30 seconds.

  - Pass (“Good enough” in open sky):

    - It settles to a small number (often 0–6 ft) and doesn’t jump around wildly while you’re standing still.

  - Fail:

    - It swings by ~10+ ft repeatedly while you’re not moving.

- Step 3 — “5-ft ring” check (45 seconds):

  - Walk ~15–20 ft away from TEST-GPS.

  - Walk back toward it slowly until you hit ~10 ft.

  - Now do a small circle around the point (radius ~6–10 ft) and watch the distance number.

  - What you’re looking for:

    - You can find a spot where the distance reading reaches a clear minimum, and if you stop there it stays ≤5 ft for ~10 seconds.

  - If you can do that in open sky, your phone/onX combo is behaving normally.

- Step 4 — Canopy reality check (15 seconds, optional but helpful):

  - Walk under a couple trees (or near the house) and repeat the 30-second stability check.

  - If the number becomes jumpy here, that’s your sign that on the property boundary (under hardwood canopy) you should rely more often on the stake-at-minimum-distance method when it won’t settle.

- Your “GO / NO-GO” rule for The Fort boundary work

  - GO (tree OK): you can get ≤5 ft and hold it ~10 seconds.

  - NO-GO (stake/post): after ~2 minutes near the waypoint it won’t settle or it keeps bouncing while you stand still.

### Phone 14 tweaks (do these once):

- Precise Location ON (must)

  - Settings → Privacy & Security → Location Services → onX → Precise Location ON + While Using (or Always).

- Turn OFF Low Power Mode

  - Settings → Battery → Low Power Mode OFF (use a battery pack instead).

- Keep Wi-Fi ON (even with no service)

  - Control Center → Wi-Fi ON.

  - (Doesn’t need to connect; it can still help the phone’s positioning stack.)

- Don’t pocket the phone on final approach

  - Hold it in your hand for the last 30 ft. Your body can block signal.

- Let it “warm up”

  - Open onX and leave it running 60 seconds before you start walking the line.

### 2-minute “woods calibration” for iPhone 14 + onX:

Do this once at the property (or anywhere with similar canopy/terrain), before you start painting.

- Step 1 — Pick a simple reference point (30 seconds):

  - Stand next to something you can physically identify again: a fence corner, gate post, driveway edge, or a distinct tree.

  - Drop a waypoint in onX named CAL-1.

- Step 2 — Settle test (45 seconds):

  - Tap CAL-1 → Go To.

  - Stand still for 45 seconds, phone held chest-high, screen on.

  - Watch the distance-to-waypoint.

  - GOOD ENOUGH under canopy looks like:

  - It settles and stays mostly within a tight band (you’ll see small movement, but not huge swings).

  - You can get it to ≤5 ft and keep it there ~10 seconds at least once.

  - NOT GOOD ENOUGH looks like:

  - While standing still, it repeatedly jumps 10+ ft, or never gets close and never stabilizes.

- Step 3 — Repeatability test (45 seconds)

  - Walk ~25–30 ft away (count ~12–15 normal steps).

  - Walk back to CAL-1 and stop where the distance is minimal.

  - If the minimum spot is roughly the same place you started (within a few feet), that’s a good sign.

- Your “paint vs stake” decision rule (simple):

  - Paint a tree only when you can achieve ≤5 ft and hold ~10 seconds.

  - If not: set a stake/post at the minimum-distance spot (don’t hunt far for a tree).

Field Card — Paint Posting Markers (ASoT 90-ft Waypoints)

Goal

Hit each waypoint PNT-### in order. Place a mark on the nearest tree within 5 ft; if none (or GPS isn’t stable), place a stake/post at the best-minimum spot.

0) Start-of-Day Setup (2 minutes)

- iPhone: Low Power Mode OFF

- Settings → Location Services → onX: Precise Location ON

- Keep Wi-Fi ON

- Open onX and let it run 60 seconds before walking

- Carry: flagging (optional), paint, stake/post kit, hammer, marker/Sharpie, phone battery pack

1) For each waypoint PNT-### (repeat)

A) Navigate

- Open PNT-### → Go To

- Walk until you’re within ~30 ft

B) Tight approach

- Tap the location button to center on your blue dot

- Tap again for Compass Mode

- Slow down; hold phone in hand, chest-high

C) “Good Enough” GPS test (do this every point)

- At ~10–15 ft, STOP for 20–30 seconds

- Watch the distance-to-waypoint:

✅ GOOD ENOUGH

- You can reach ≤5 ft and keep it ≤5 ft for ~10 seconds

❌ NOT GOOD ENOUGH

- While standing still, distance jumps around (often 10+ ft) or won’t settle after ~2 minutes

D) Place the mark

- If GOOD ENOUGH:

  - Find closest suitable tree within 5 ft

  - Paint (and flag if you want)

- If NOT GOOD ENOUGH (don’t gamble):

  - Walk a small 6–10 ft circle around the point

  - Find the lowest distance value

  - Set stake/post there and paint it

2) Log it (30 seconds)

- In onX waypoint notes: PAINTED or STAKE, date

- Take a quick photo of the tree/stake (optional but highly recommended)

3) If you hit a “problem stretch”

(heavy canopy, steep hollows, GPS unreliable)

- Use stakes more often

- If you can’t confidently meet the ≤5 ft rule for multiple consecutive points, pause and consider adding an extra marker later (tighter spacing set) for that segment.